Strata® CIX™

Product Bulletin

PB-0186

September 26, 2012

# The Evolution of the Strata Product Line

## Introduction

The information in this bulletin is provided as a reference to compare past and present versions of Strata products. This information will assist you in determining what features and options can be added to your existing customer base of Strata systems. It will help explain to your Strata customers what capabilities they have in their systems compared to current products, and what they gain when upgrading from older to newer Strata systems.

This information will also enable you to show prospective customers the ongoing progression of enhancements and new product development within the Strata product line. This is proof of Toshiba's commitment to an evolving product line strategy, keeping Strata products up with developing trends and new requirements in the telecommunication system industry.

## Strata Product History

1978 – 1981:

- The Strata product line has a long and rich history. It started back in 1978 with the Strata III, Toshiba's first electronic, stored program key system. The Strata III was one of the first “skinny wire” systems on the market and led the way to the development of future Strata products.

- With each new product model, and each new software version, Toshiba has continually enhanced features and capabilities.

1982 – 1985:

- The Strata VI first appeared in 1982 and was Toshiba's first microprocessor-based electronic key system.

- It set the stage for development of a family of Strata systems, with the introduction of the smaller Strata S, and larger Strata XII and XX in 1983.

1986 – 1988:

- The Strata "e" release brought additional capabilities to the Strata S, VI, XII, and XX systems in June 1986.

- In March 1988, Release 2 brought more new capabilities to the Strata Se, VIe, XIIe, and XXe, making Strata one of the most feature rich families of hybrid/key systems on the market.

1989:

- In the late 1980's, digital technology began to enter the key systems market.

- Toshiba's first digital key telephone system was the Strata DK series in 1989. The DK56 and DK96 replaced the XIIe and XXe in the Strata product line and set the standard architecture for our digital product line.

1990:

- Strata DK Release 2 brought many important enhancements to the DK56 and DK96 systems.

- At that same time in 1990, Toshiba also introduced the DK24, bringing digital technology farther down the size range of the Strata product line. The DK24 also filled an important need for a system sized between the 6x16 Strata VIe and the Strata DK56.

1991 – 1992:

- Strata DK Release 3 in 1991 introduced Strata DK24/56/96 systems to digital telephones and data switching, both for the first time in the Strata product line.

- Then, in 1992, Release 4 brought even more new capabilities to DK24/56/96 systems, which included new and improved digital telephones with LCD feature prompting and soft key operation.

- In November 1992, the DK16 replaced the Strata VIe in the Strata product line, bringing digital technology to small system users. With all the features and capabilities of larger Strata DK systems, the DK16 provided sophisticated functionality rarely found in a small hybrid/key system.

1993:

- The Strata DK8 followed closely behind the DK16, completing the transition of entire Strata product line to digital technology. With its introduction in April 1993, the DK8 replaced the Se in the Strata product line, bringing powerful features to small system users, rarely found in systems of this size.

- In June 1993, Toshiba introduced another milestone product to the hybrid/key systems marketplace, the Strata DK280 Modular Digital Telephone System. The DK280 replaced the DK24, DK56, and DK96 in the Strata product line. This important new product provided many advanced new features, significantly expanded the size of Strata systems, and made modular growth and migration more cost effective than ever before.

1994 – 1995:

- Toshiba introduced Release 2 of the Strata DK280 in June 1994. Release 2 provided ACD/MIS, Attendant Console, SMDI voice mail interface, and Pooled Line Key improvements.

- This provided the most complete call answering solution and flexibility available, and many new call center application opportunities.

1996:

- Toshiba introduced Release 3.0 of the Strata DK280 in 1996. Release 3.0 provided Multiple DNs, Caller ID/ANI, DNIS, and other important advanced hybrid features.

- At the same time, Computer Telephony Integration TAPI capabilities became available for all Strata DK systems, opening the door to many exciting new custom PC-driven applications.

- Later in the year, the DK16e replaced the DK16 adding Caller ID and many new enhancements.

- Also in 1996, the Digital Cordless Telephone was introduced.

1997:

- The entire Strata DK product line was updated in 1997. The DK14, DK40, and DK424 replaced the DK8, DK16e, and DK280, expanding system size and adding feature enhancements.

- Release 3.1 added more new features to the DK424.

- The PC Attendant Console improved operator functionality, Call Center Viewer provided more ACD display options, and StrataLink provided important CTI enhancements.

2 of 20

1998:

- New product releases in 1998 included the PC Digital Telephone, enhancements to StrataLink CTI capabilities, enhancements to the PC Attendant Console, the Strata AirLink Wireless Telephone System external interface version, the Hospitality Management Information System (HMIS), and the Single Line Digital Telephone.

- The DK424 Release 3.2 added feature improvements and the DK424 Release 4.0 added ISDN Primary Rate Interface (PRI) capabilities and other new feature enhancements.

1999:

- The Strata DK40i added ISDN Basic Rate Interface (BRI) S/T interface capabilities to our most popular small system and Release 4.1 software added ISDN BRI S/T interface capabilities to the Strata DK424.

- Other new product releases included the Toshiba Ultimate Communicator, Strata AirLink Wireless integrated interface version, Voice over IP gateway products, Insight DK MIS reporting packages, new HMIS software packaging options, and StrataControl end user administrative software.

2000:

- The year 2000 brought many new developments to the Strata product line. Release 4.15 provided enhancements to the ISDN PRI capabilities in the Strata DK424. Release 4.3 provided more enhancements to both PRI and BRI and other important new features for the Strata DK424.  Release 4.3 also enhanced the RCTUA processor to support ACD, T1, and PRI.

- The Strata DK424i provided new system hardware architecture that would later be compatible and upgradeable to the larger Strata CTX670 system.

- The DK40i Release 4.3 provided ACD, ISDN PRI, BRI U interface, and T1 interface, bringing additional powerful big system functionality to smaller users.

- Important new adjunct products were added to the Strata product line. The Toshiba IP Voice Gateway added another important Voice over IP product in the form of a card that integrates into a slot in the Strata DK cabinet. MCK Branch Office EXTender products provided digital telephone extensions to remote office locations. The Hospitality Management Information System (HMIS) Release 2 added Property Management System (PMS) Interface and several new features that increase functionality and value.

- An additional model digital cordless telephone was introduced, and a full duplex speakerphone model was added to the 2000-series digital telephone product line.

2001:

- MCK IP EXTender products provided digital telephone extensions to individual remote users via IP network connection.

- Insight DK Lite provided low cost call center supervisor MIS display capabilities for small ACD applications.

2002:

- Toshiba introduced Strata CTX100 and CTX670 digital business telephone systems, with the Strata CTX100 replacing the DK40i and the CTX670 replacing the DK424i in the Strata product line, bringing larger capacity, multi-system networking, enhanced voice mail integration, and many other important new features.

- 3000-series digital telephones represented the third generation of digital telephones from Toshiba, providing many enhancements and a more modern design. The CTX Attendant Console provided efficient high-volume call answering.

- Also in 2002, powerful ACD capabilities were added to the Strata CTX, with the ACD server bringing advanced call center features and the OAISYS suite of value-added applications. Insight products complimented sophisticated ACD functionality with equally sophisticated call center reporting applications.

2003:

- Important wireless developments included SpectraLink wireless telephones added to the Strata product line. Two new models of digital cordless telephones, DKT2204-CT and DTK2304-CT, replaced the two original versions, DKT2004-CT and DKT2104-CT. The SoftIPT soft phone client software was introduced to provide all the features of desk telephones to laptop PCs connected to wireless local area networks.

3 of 20

The Strata CTX670 Release 1.3 software provided important multi-system networking enhancements.  Release 2.0 IP-enabled Strata CTX systems, providing support for Toshiba IPT1020-SD IP telephone connection. Release 2.1 provided multi-system networking via IP networks.

- The Strata CTX100-S was introduced as a competitively-priced small system solution.

2004:

- Release 2.2 provided more networking and general feature enhancements to Strata CTX systems, and Release 2.22 provided Spanish LCD display, PRI Call-by-call Min./Max., Caller ID over Strata Net, and Attendant Console overflow to multiple call groups.

- New versions of ACD 2.0 and Insight 3.12 software enhanced call center functionality.

- Strata CTX670 rack-mountable cabinets were introduced to provide additional installation flexibility.

- Toshiba’s second generation of IP telephones, the 2000-series, was introduced with three new models and Info Manager applications.

- The Strata CTX28 was introduced for small office users. The CTX28 replaced the DK14 in the Strata product line adding built-in Caller ID, additional features, expanded capacity, and an integrated voice mail (GVMU) card with Call Record, Call Monitor and Retrieve, and LCD soft key prompting of voice mail features.

2005:

- The Strata CIX200 with Release 3.0 software represented Toshiba’s second generation IP system with peer-to-peer IP communication of MEGACO+ and Uniden UIP200 SIP telephones.

- The Strata Media Application Server (MAS) provided IP integration to the Strata CIX for support of voice mail, ACD, and other value-added applications all on one platform.

- SMC products provided Power-over-Ethernet (PoE) capabilities and SonicWALL products provided network security for Strata CIX IP systems.

- SoftIPT soft phone client software was added for PDAs, enabling both PDAs and laptop PCs to connect to wireless local area networks.

- Release 3.1 brought IP peer-to-peer functionality to Strata CTX100 and CTX670 systems, offering a migration path to turn CTX into CIX systems, with all the same functionality of the CIX200, including FeatureFlex and MAS integration. Release 4.0 added UCD, Centralized Network SNMP, Traffic Measurement and Reporting, SMTP Traps for Alarm Notification, and Dial by Name Directory on Small LCD to Strata CIX systems.

- 3200-series digital telephones replaced the 3000-series with new styling similar to Toshiba IP telephones. Strata CTX28 was enhanced with Release 2.22, adding Spanish LCD prompts.

- The Stratagy iES16 was added to the Toshiba voice mail product line. Strategy ES Release 4.6 provided enhanced Unified Messaging IMAP integration, Outlook XP/2003 compatibility, and SoftFax.

2006:

- Strata CIX Release 4.1 added VLAN support, BCOCIU and BSTCIU Caller ID cards, and Strata Net per-port licensing. HotDesk, a FeatureFlex application, introduced cost-effective digital telephone operation for a shared office environment.

- The PC Interface Unit (BPCI) and TAPI driver software provided Outlook integration for digital telephones.

- The CIX Attendant Console was made available as a software-only License, enabling users to provide their own PC.

- An additional IP telephone model (IPT2010-SDC) was introduced with analog loop-start CO line connection for local dialing.

- ACD Release 2.1 brought skills-based routing enhancements, and myTASKE Reporter introduced a comprehensive suite of web based reporting tools designed specifically for the smaller contact center.

- The Video Communication Solution (VCS) was introduced with point-to-point video conferencing and desktop collaboration capabilities including desktop/application sharing, file transfer, and message board.

- A new 2U hardware chassis became available for the Strata Media Application Server (MAS), replacing the 1U platform with some important enhancements including front-side cable connections, 2 full length board slots, improved cabinet cooling fan configuration with reduced operational noise, shorter cabinet length for easier rack installation, and RoHS compliance (Restricted use of Hazardous Substances) environmental directive.

- The Strata CIX40 was introduced for small office users, completing the transition of entire Strata product line to IP technology. The CIX40 replaced the CTX28 in the Strata product line adding support for IP telephones, Strata Net networking, expanded capacity, and an enhanced integrated voice mail (GVPH) card with built-in modem.

2007:

- Strata CIX Release 4.2 provided increased Caller ID Storage telephone capacity, CSTA enhancements, and additional SIP telephone features Hold, Conference, and Transfer. Taking advantage of these additional SIP features, the Hitachi WIP5000E-A wireless IP telephone was introduced to enhance mobility via wireless LAN connection to the Strata CIX.  Also with Release 4.2, a new version of eManager provided enhancements including integration with DESI® keystrip printing program, import/export of DID/DNIS assignments, station directory, station and system speed dial names, and numbers and group upload of network calling numbers.

- Strata CIX Release 5.0 added the IP Hot Desk feature with IP User Mobility to automate the device login/logout procedure throughout the network, and Release 5.1 added SIP Trunking.

- Strata ACD R2.1 with Unifier provided networked ACD of multiple call center sites to operate as one integrated call routing system.

- The Video Communication Solution (VCS) Release 2.0 enhanced 2-party video to 3-party video conferencing.

- The MicroMAS provides all of the features as the larger MAS but in a much smaller size and price point for smaller customers.

- The LVMU1A replaced the IVP8 in the voice mail product line with increased message storage and new features including Call Record and LCD Soft Key feature prompting, and built-in modem.

- A FeatureFlex feature, Simultaneous Ringing, enabled calls to ring at both your desk telephone and your cell phone, so you can answer the call from either of the ringing telephones.

- Strata CIX40 Release 2 increased system capacities from 6 to 11 trunks, from 8 to 24 IP channels, and from 4 to 6 or 8 voice mail ports.

- The 5000-series Digital Telephones replaced the 3200-series with new styling and new models that include backlit LCD and LCD key labels.

2008:

- The 2U chassis model of Strata Media Application Server (MAS) was enhanced to support fax applications by electronically activating licensing for this feature group option and adding up to two fax modem boards to provide from 1-16 channels of Stratagy soft fax.

- The centralized administration function of eManager was enhanced to better support networked applications with the ability to manage a single site or a group of CIX systems simultaneously in a single session. Active Directory Services were added to synchronize databases and speed installation.

- The SoftIPT version 2.2 provided Vista operating system support, G.729 and echo cancellation ehancements.

- Net Phone version 6.0 enhancements included CIX Presence.

- The 5000-series IP Telephones replaced the 2000-series with new styling and new models that include backlit LCD and LCD key labels.

- Additional SIP Trunking compatibility certification was added and the list now includes American Broadband Services (ABS), AT&T, Broadcore, Cbeyond, Clearfly Networks, IPtimize, Paetec/McLeod USA, and Twist Networks.

- The CIX1200 was added to the Strata family, expanding maximum capacity up to 440 trunks, 1,000 telephones, and 1,152 ports total.

- The DP5130-FSDL full-duplex digital telephone model was added to the 5000-series family of digital telephones.

- Strata CIX R5.2 adds integration with Microsoft Exchange to provide unified messaging using Exchange 2007 as the voice mail server.

2009:

- Two Nurse Call Systems (Cornell Aura 2.0 and IgeaCare) were introduced to provide comprehensive, integrated nurse response and reporting to compliment the capabilities of the Strata CIX for assisted living facilities.

- Strata ACD version 1.6 provided scalability enhancements to support up to 20 Strata CIX nodes for network ACD applications. Multimedia Web Callback was also added to provide website visitors access to the contact center from the web site by leaving a callback request in the queue.

- The mobility portfolio was enhanced with additional models of Polycom SpectraLink wireless IP telephones (8002/8020/8030) that operate via wireless LAN connection to the Strata CIX, Polycom Kirk SIP DECT wireless telephones, and the Toshiba DKT2404-DECT cordless telephone.

- Mobility was further enhanced with the introduction of uMobility, a fixed mobile convergence dual-mode smart cell phone application providing on-premise WLAN functionality and off-premise cellular coverage.

- The centralized administration function of Network eManager was enhanced to speed installation with an Excel import utility that can populate program data from an Excel spreadsheet containing names, phone numbers, email addresses, etc. and works in conjunction with Network eManager’s Active Directory Services utility.

- Microsoft Exchange 2007 R2 integration with the Strata CIX provided remote call control and enhanced telephony presence to the OCS client application.

- The Toshiba Unified Communications Suite bundled several popular Toshiba UC applications into a package and UC client license that is easy to purchase and implement.

- Stratagy View is a Web-based unified messaging solution enabling users to access their voice and fax messages residing in their voice mailboxes from anywhere there is Internet access using only an Internet browser.

- The Stratagy iES16 was enhanced with increased RAM and Compact Flash storage to provide support for the XP Embedded OS, increase of voice storage from 100 to 150 hours, and compatibility with Network eManager for unified CIX/voice mail system programming.

- Strata Messaging brought low-cost 2-8 port unified messaging on a Linux PC appliance platform.

- Additional SIP Trunking carrier compatibility certification was added and the list now includes American Broadband Services (ABS), AT&T, Broadcore, Broadvox, Cbeyond, Clearfly Networks, IPtimize, MM Internet, NTELOS, Paetec, Twist Networks, and XO Communications.

- Survivability for IP telephones on Strata CIX was enhanced through the use of AudioCodes gateways. Both outgoing and incoming calls automatically follow the IP telephones to their alternative location on a secondary Strata CIX system. Voice mail on MAS or MicroMAS will still be operational even when one of either the primary or secondary Strata CIX is completely down because the MAS can be simultaneously registered to both Strata CIX systems.

2010:

- Strata Call Manager brought a new look and feel with enhanced functionality to the Net Phone unified communications client application.

- Strata Meeting added on-demand and scheduled meet-me audio conferencing capabilities for advanced applications requiring more than the basic 8-party conferencing provided within Strata CIX.

- The mobility portfolio was enhanced with the addition of the Motorola EWP series SIP wireless smart phones. The uMobility fixed mobile convergence mobility product was enhanced with support of Apple iPhone devices, RIM Blackberry devices, and Google Android devices.

- The Strata Messaging low-cost unified messaging system was enhanced with the addition of a Fax option, making it the most robust UM solution for small systems.

- Strata ACD version 2.4 provided additional multimedia contact center enhancements to support live Web Chat with contact center agents for visitors accessing the contact center from the web site.

- Strata CIX40 Release 3 hardware modifications to the cabinet and power supply brought improved power supply operation and grounding.

- Additional SIP Trunking carrier compatibility certification was added and the list now includes AireSpring, American Broadband Services (ABS), AT&T, Broadcore, Broadvox, Cbeyond, Clearfly Networks, Everest Broadband, Level 3, Line Systems, MM Internet, New Global Telecom, NewRoads Telecom, NextLevel Internet, NTELOS, Paetec, Saddleback Communications, Select Connect Communications, Smart Choice/All SIP, SoTel Systems, TelePacific Communications, Twist Networks, Vinculum Communications, Xchange Telecom, and XO Communications.

2011:

- The IP5000-series family of IP telephones was enhanced with the addition of four new models to provide additional choices and lower-priced models.

- Caller ID enhancements and Call Waiting Swap on Analog single line telephones improved analog telephone operation.

- Strata Meeting on-demand and scheduled meet-me audio conferencing was enhanced with the addition of web collaboration capabilities.

- The mobility portfolio was enhanced with the addition of the Toshiba IP4100 SIP DECT wireless telephone.

- The Call Manager version 7.5 added a VoIP soft phone that emulates IP5000 desk phone operation, slim docking mode, notification pop-up for incoming calls and alarms from the taskbar, and other useful enhancements.

- Additional SIP Trunking carrier compatibility certification was added and the list now includes 123.net, ACD Telecom, AireSpring, American Broadband Services (ABS), AT&T, Broadcore, Broadvox, Cbeyond, Clearfly Networks, Everest Broadband, Integra Telecom, IPtimize, Level 3, Line Systems, MM Internet, New Global Telecom, NewRoads Telecom, NextLevel Internet, NTELOS, Paetec, Saddleback Communications, Select Connect Communications, Smart Choice/All SIP, SoTel Systems, TelePacific Communications, Time Warner Telecom, Twist Networks, Vinculum Communications, Xchange Telecom, and XO Communications.

2012:

- The mobility portfolio was enhanced with the addition of the Polycom 8400-series wireless telephones that operate over the customer’s wireless LAN.

- Additional SIP Trunking carrier compatibility certification was added and the list now includes 123.net, Access One, ACD Telecom, AireSpring, American Broadband Services (ABS), Appia, AT&T, Broadcore, Broadvox, Cbeyond, Clearfly Networks, CTC Connect, Everest Broadband, Integra Telecom, IPtimize, Level 3, Line Systems, MM Internet, Momentum, New Global Telecom, NewRoads Telecom, NextLevel Internet, Nextera, NTELOS, Optimum Business, Paetec, Saddleback Communications, Select Connect Communications, Smart Choice/All SIP, SoTel Systems, TDS, TelePacific Communications, Time Warner Telecom, Twist Networks, Vinculum Communications, Xchange Telecom, and XO Communications.

As you can see, Toshiba has been busy keeping the Strata CIX product line up to date and growing in capabilities, and we will continue to do so on an ongoing basis.

## Strata Version and Product Release History Chart

With each new product model, and each new software version, Toshiba has continually enhanced features and capabilities of Strata telecommunication systems.

Strata DK24/56/96, DK8, DK16

| Software Version | Release Date | Enhancements |  |
| --- | --- | --- | --- |
| DK56 DK96 | May 1989 | Strata DK56 replaced Strata XIIe R2 Strata DK96 replaced Strata XXe R2 |  |
| Release 1 | May 1989 | Digital system technology Flexible port assignment (lines x stations) Fixed destination call forward Direct inward station calling Power failure transfer to standard telephone | Non-blocking intercom paths Intercom hold Standard station compatibility DTMF/DP External zone paging Selectable internal call ring tones |
| DK24 | July 1990 | DK24 added to Strata product line | DK24 same R2 features as DK56/96 |
| Release 2 | July 1990 | Call park Call forward – busy Call forward – no answer Call forward – busy/no answer Direct inward system access (DISA) Release button | Intercom user name display on LCD Selective call pickup Distinctive ringing on standard telephones Increased door lock capacity from 1 to 5 Increased music interfaces from 2 to 3 Additional SMDR (toll call) report |
| Release 3 | March 1991 | Digital telephones & data switching Verifiable account codes “0+” credit card dialing Traveling class of service | Privacy button Night ringing over selected page zones Busy station LCD messaging Group station LCD messaging |
| Release 4 | September 1992 | LCD feature prompting with soft keys Multi-step feature buttons Automatic hold Call forward block on hands-free calls | Busy station transfer/ringing Separate BGM/MOH source Programmable door phone ringing Programmable SMDR call threshold time |
| DK16 | November 1992 | DK16 replaced VIe R2 in Strata product line | Digital technology & DK R4 features |
| DK8 | April 1993 | DK8 replaced Se R2 in Strata product line | Digital technology & DK R4 features |

Strata DK280/424/424i, DK14, DK16e/DK40/DK40i

| Software Version | Release Date | Enhancements |  |
| --- | --- | --- | --- |
| DK280 | June 1993 | DK280 replaced DK24/56/96 systems | Modular growth |
| Release 1.0 | June 1993 | Modular multi-cabinet architecture Expanded capacity to 280 ports Automatic station relocation Automated attendant Call forward – external destination Call forward – remote change destination Call pickup for multiple station groups | Direct inward dialing (DID) T1 interface Ground-start CO line support Tie line transfer recall 20-button Add-on module Expanded ring tones for distinctive ringing Continuous DTMF signal time |
| Release 2.0 | June 1994 | Automatic call distribution & reporting Attendant Console | SMDI voice mail interface Pooled line key enhancements |
| Release 3.0 | March 1996 | Multiple directory numbers Caller ID & ANI DNIS TAPI support Release/answer button | Message waiting on standard telephones Camp-on tone for standard telephones Call park orbits, auto park, park/page Off-hook call announce – handset mode 10-digital local dialing |
| DK16e | December 1996 | DK16e replaced DK16 in Strata product line | Caller ID & DK280 R3.0 features |
| Release 3.1 | February 1997 | Distributed hunting |  |
| DK424 A,B,C/D | February 1997 | DK424 replaced DK280 in Strata product line | A, B, C/D processors provided same capacity |
| DK424 E/F | July 1997 | E/F processor option added | Expanded system capacity to 424 ports |
| DK40 | September 1997 | DK40 replaced DK16e in Strata product line | Expanded capacity & DK424 R3.1 features |
| DK14 | December 1997 | DK14 replaced DK8 in Strata product line | Expanded capacity & DK424 R3.1 features |
| Release 3.2 | May 1998 | System open-architecture interface Voice mail control during conference call | Selective pickup of all call page Call transfer music or ringing option |
| Release 4.0 | November 1998 | ISDN Primary Rate Interface (PRI) |  |
| Release 4.1 | March 1999 | ISDN BRI S/T interface | VoIP trunk gateway |
| DK40i | May 1999 | DK40i replaced DK40 in Strata product line | ISDN BRI & DK424i R4.1 features |
| Release 4.15 | March 2000 | ISDN PRI call-by-call enhancements |  |
| Release 4.3 | June 2000 | ISDN BRI U interface Variable capacity (fractional) PRI | E911 integrated interface E911 over ISDN PRI trunks |
| DK424i | June 2000 | DK424i replaced DK424 in Strata product line | New cabinets and processors CTX compatible |

Strata CTX

| Software Version | Release Date | Enhancements |  |
| --- | --- | --- | --- |
| CTX100 CTX670 | February 2002 | CTX100 replaced DK40i in Strata product line CTX670 replaced DK424i in Strata product line | Expanded system capacity to 112 ports Expanded system capacity to 670 ports |
| Release 1.0 | February 2002 | Multi-system networking (QSIG) via PRI Advanced ACD capabilities Voice mail LCD feature prompting/control Call record to voice mail Transfer direct to voice mailbox Voice mail conference Caller ID call history & redial Hot dialing Lost call treatment System-wide call forwarding default | High-speed 32-bit RISC processor SmartMedia card database storage/upgrade Digital PAD transmission level control CSTA system CTI link System fault finding & diagnostics 8-party conference & split conference Night transfer auto scheduled Stutter dial tone Through dialing from station |
| Release 1.3 | March 2003 | DSS/BLF networking enhancements Centralized voice mail enhancements | Multiple call/delayed ringing Distinctive ringing enhancements |
| Release 2.0 | April 2003 | IP telephone support |  |
| CTX100-S | July 2003 | CTX100-S added to Strata product line | Low-priced processor for small configurations |
| Release 2.1 | July 2003 | Multi-system networking via IP |  |
| CTX670 RM | February 2004 | Rack-mount cabinets available for CTX670 |  |
| Release 2.2 | March 2004 | Network DN tables across network nodes DISA calling across tie lines Expanded number of night transfer buttons | Tenant service up to 8 tenants Conference split/join Enhanced CSTA link & new ACD features |
| CTX28 | July 2004 | CTX28 replaced DK14 in Strata product line | Caller ID built-in, expanded capacity, GVMU |
| Release 2.22 | October 2004 | Spanish LCD display PRI Call-by-call Min./Max. | Caller ID over Strata Net Attendant Console overflow to multiple groups |

Strata CIX

| Software Version | Release Date | Enhancements |  |
| --- | --- | --- | --- |
| CIX200 Release 3.0 (CIX200 only) | February 2005 | CIX200 added to the Strata product line Release 3.0 software for CIX200 only Peer-to-peer IP communication | FeatureFlex adaptability Unified Administration (CIX & SES) Personal Administration (My Phone Manager) |
| MAS 1U SES Release 5.0 (MAS only) | February 2005 | Media Application Server introduction All value-added applications on one platform 1U chassis server design | Automated Configuration Wizard UM enhanced with IMAP synchronization Spanish voice prompts (neutral dialect) ASR multi-level, speech-enabled menus |
| Release 3.1 | May 2005 | CTX100 rebranded the CIX100 CTX670 rebranded the CIX670 Peer-to-peer IP communication on CIX100/670 | IPT2008-SDL compatibility Info Manager applications |
| Release 4.0 | November 2005 | Traffic Measurement & Reporting SMTP Traps for Alarm Notification Centralized Network SMDR | Uniform Call Distribution (UCD) Dial by Name Directory on Small LCD SD card media database storage/upgrade |
| Release 4.1 | June 2006 | VLAN Support Support for BCOCIU, BSTCIU Caller ID cards | Strata Net per-port licensing |
| MAS 2U | August 2006 | 2U server chassis replaced 1U chassis Shorter cabinet length easier rack installation Reduced fan noise | Front-side cable connections 2 full length board slots RoHS compliance |
| CIX40 | August 2006 | CIX40 replaced CTX28 in Strata product line | Strata Net networking, IP telephones, GVPH |
| Release 4.2 | January 2007 | Additional SIP features (hold, transfer, conf.) CLID Storage telephone capacity increased | Caller Name on CSTA events Divert enhancement on CSTA |
| MicroMAS | June 2007 | Capabilities of the larger MAS but in a smaller size and price point for smaller customers | Fax feature group supported |
| Release 5.0 | September 2007 | IP Hot Desking and IP User Mobility |  |
| CIX40 Release 2 | November 2007 | Expanded CO line capacities from 6 to 11 | Expanded IP capacities from 8 to 24 channels |
| Release 5.1 | December 2007 | SIP Trunking |  |
| Release 5.2 | September 2008 | Microsoft Exchange 2007 UM integration |  |
| ACD Web Callback | August 2009 | Multimedia Web Callback enhancement to contact center capabilities |  |
| Survivability | November 2009 | Survivability of IP telephones to backup systems |  |
| BSTU1A & BSTCIU1A | December 2009 | New Analog Cards with improved Disconnect Supervision |  |
| Release 5.2 MT037 | January 2010 | Analog SLT Caller ID Enhancements |  |
| Strata Call Manager | January 2010 | Next generation Net Phone with new look and enhanced functionality |  |
| CIX40 Release 3 | June 2010 | Improved power supply operation and grounding |  |
| ACD Web Chat | November 2010 | Multimedia Web Chat enhancement to contact center capabilities |  |
| Release 5.2 MT051 | January 2011 | Call Waiting Swap Enhancements |  |
| Strata Meeting | April 2011 | Meet-me Audio Conferencing with Web Collaboration |  |

## Voice Messaging Version and Product Release History Chart

With each new product model, and each new software version, Toshiba has continually enhanced features and capabilities of Stratagy and other voice messaging systems.

| Software Version | Release Date | Enhancements |  |
| --- | --- | --- | --- |
| Release 1.10 | October 1994 | Original Stratagy software version |  |
| Release 1.11A | February 1995 | Recognition of “A tone” answer supervision Enhanced supervised transfer voice cut through | Intermittent Fax Tone Detection |
| New Stratagy voice boards | July 1995 | New half-sized Rhetorex voice boards Existing boards modified for compatibility | New advanced architecture |
| Release 1.21 | February 1996 | Introduced Stratagy Activation Module Reduced disconnections from feature collision | Improved Fax application functionality Quicker/easier Fax application installation |
| Release 2.09 (Stg 4 & Stg 24) | June 1996 | Enhanced user interface New and Saved message queues | Hot Zero LIFO/FIFO message order |
| Release 2.09E (Stg 6 & 6 Lite) | June 1996 | Rhetorex Duet (2-port) boards supported Rhetorex Quartet (4-port) boards supported |  |
| Release V4.2B Stratagy 4 Lite | August 1996 | New low-cost 4 Lite platform introduced Admin programming utilities added |  |
| Release Vx.212 (Stg 4, 6, 6 Lite) | December 1996 | Enhanced shut down procedure Enhanced Defrag performance |  |
| Release V4.2E (Stg 4 Lite only) | December 1996 | Plug and Play for Strata systems Increased communication rate to 9600 bps Manual dialing mode for remote access | Enhanced shut down procedure Enhanced Defrag performance |
| Release VDK.2G Stratagy DK | May 1997 | New Stratagy DK “in-skin” platform introduced Enhanced Admin menu-driven interface | Plug and Play for Strata systems Backup and Restore utilities |
| Hospitality Application | August 1998 | Guest room messaging works with HMIS Also works with any hotel/motel program | Resets call forwarding on guest room phone Call forward enabled from front desk |
| Release V4.2N Release Vxx.225 | September 1998 | Year 2000 compatibility AMIS Networking | Daylight savings time parameter Messaging compliant improvements |
| Release VDK.2N (Stg DK only) | September 1998 | Year 2000 compatibility AMIS Networking Park head feature | Daylight savings time parameter Messaging compliant improvements |
| Release 3.0 Stratagy Flash | February 1999 | New low-cost 4 Flash platform introduced Uses solid-state flash memory technology | Plug and Play for Strata systems Pending messages |
| Stratagy DK voice driver 1.07 | May 1999 | Increased voice storage to 85 hours Improved BIOS |  |
| SES Release 1 | May 1999 | Interactive Voice Response (IVR) SES platform Two platforms available (Model A & Model B) | IVR feature group |
| Stratagy 24 Plus | June 1999 | New larger platform introduced | All PC circuitry on single board computer |
| Stratagy 24 SBC | September 1999 | Replaced Stratagy 24 | All PC circuitry on single board computer |
| Stg 6 Lite SBC | November 1999 | Replaced Stratagy 6 Lite | All PC circuitry on single board computer |
| Release 3.1 | November 1999 | Year 2000 compliance for all Stratagy models AMIS Networking (all models except Flash) Automatic system recovery Enhanced shutdown procedure Optional system wide name announcement Improved prompting for forwarding messages | New message destination selection Pending messages for all models Stratagy Advisor Tutorial Dialogic voice boards added (ProLine/2V, Dialog/4, D/160SC-LS) |
| Stratagy 6D Stratagy 24D | November 1999 | Stratagy 6D replaced 6 Stratagy 6D has internal modem & 1 serial port | Stratagy 24D replaced 24SBC Stratagy 24D has 4 serial ports |
| SES Release 2 | February 2000 | Messaging Voice Procesg. (MVP) SES platform Two platforms available (Model A & Model B) | Fax feature group |
| Hospitality Enhancement | March 2000 | Hospitality application compatible with Stg DK Compatible with all PC-based Stratagy models |  |
| SES UM & TTS | July 2000 | Unified Messaging & Text-to-Speech for SES | UM & TTS feature groups |
| SES ASR | September 2000 | Automatic Speech Recognition for SES | ASR feature group |
| Release 3.2 | November 2000 | DTMF Guard Timer Disk Full Prevention | New User Tutorial Enhancement AMIS maximum message length |
| Release 3.3 | January 2002 | AMIS Networking | Auto Schedules for Daylight Savings KSU Time sync between VM & CTX |
| Stratagy IVP8 | January 2002 | New IVP8 “in-skin” platform introduced | Flash storage & Release 3.3 features |
| SES Release 3 | February 2002 | Model C1 lower cost platform added VM LCD feature prompting/soft key operation Call Record to VM Transfer direct to voice mailbox | ASR enhancements TTS enhancements Voice mail conference |
| Stratagy iES32 | July 2002 | New iES32 “in-skin” platform introduced SoftSAM Stratagy Activation Module | SES Release 3 features Fax & UM feature groups |
| ES4, ES80, ES96, ES96R | July 2002 | SES Model C1 renamed ES4 SES Model B5 renamed ES80 SES Model A8 renamed ES96 SES Model Rack6 renamed ES96R | New models replace 6D, 24D, 24 Plus |
| IVP8 Storage Upgrade | September 2002 | Compact Flash upgrade increases storage 128MB 8 hours 196MB 12 hours | 256MB 16 hours 512MB 32 hours |
| Stratagy Flash Storage Upgrade | December 2002 | Compact Flash upgrade increases storage 64MB 6 hours 128MB 12 hours | 196MB 17 hours 256MB 22 hours 512MB 44 hours |
| SES Release 4.0 | March 2003 | Voice Mail Soft Keys Across Network Nodes Call Record to Remote Voice Mail Fax Client Enhancements | SAPI compliance for Text-To-Speech (TTS) Voice Profile for Internet Mail (VPIM) Compl. Time zone Control for Mailboxes |
| Stratagy ES48 SES 4.5 | September 2003 | Stratagy ES48 replaced ES80 New chassis with full length PCI slots | Release 4.5 specifically for ES48 only Same features as SES Release 4.0 |
| SES-8-PRO SES-48-PRO SES-96R2-SVR | April 2004 | New PC hardware platforms introduced Standardized System 1, 2, 3, and 4 servers | Also used for Strata CS, ACD, Att. Console System 4 only used for Strata CS |
| IVP8 R2 | August 2004 | Increased std storage hours from 4 to 15 hours Storage upgrade option increases to 30 hours | Faster internal model (14,400 baud) |
| MAS 1U SES Release 5.0 (for MAS only) | February 2005 | Media Application Server introduction All value-added applications on one platform 1U chassis server design | Automated Configuration Wizard UM enhanced with IMAP synchronization Spanish voice prompts (neutral dialect) ASR multi-level, speech-enabled menus |
| Stratagy iES16 | March 2005 | New iES16 “in-skin” platform introduced Compact Flash storage instead of hard drive | SES Release 4 features Fax, UM, TTS feature groups |
| SES Release 4.6 | December 2005 | UM enhanced with IMAP synchronization Updated Microsoft Outlook Form for XP/2003 | SoftFax feature with Intel voice boards |
| MAS 2U | August 2006 | 2U server chassis replaced 1U chassis Shorter cabinet length easier rack installation Reduced fan noise | Front-side cable connections 2 full length board slots RoHS compliance |
| GVMU1A | August 2006 | “in-skin” 4-port platform introduced for CIX40 |  |
| LVMU1A | May 2007 | “in-skin” 2-8-port platform introduced for CIX100/200/670 to replace IVP8 | Larger (40 hour) flash-based storage hours Call record, call monitor, soft keys, modem |
| MicroMAS | June 2007 | Capabilities of the larger MAS but in a smaller size and price point for smaller customers | Fax feature group supported |
| GVPH1A | November 2007 | “in-skin” 4-8 port platform introduced for CIX40 R2 (replaced GVMU1A with larger capacity) |  |
| SES/MAS R5.1 | January 2008 | Fax on 2U MAS with new fax board |  |
| SES/iES R4.8 | May 2009 | Stratagy View mobile unified messaging |  |
| MAS R6.0 | May 2009 | Stratagy View mobile unified messaging |  |
| iES16 6.0 & XP | October 2009 | XP Embedded operating system | Network eManager compatibility |
| Strata Messaging | November 2009 | 2-8 port unified messaging on Linux PC appliance platform |  |
| Strata Messaging R2 | April 2010 | Fax option supports up to 4 simultaneous fax transmissions |  |

## Strata CIX Upgrade Comparison Chart

There are many capabilities gained when upgrading to Strata CIX from previous generation Strata CTX and DK systems. The chart below summarizes these capabilities.

| System Features | DK424 | DK424i | CTX670 | CIX670 | CIX1200 |
| --- | --- | --- | --- | --- | --- |
| Auto Call Distribution Advanced Features & OAISYS Modules | N/A | N/A | Opt | Opt | Opt |
| Automatic Release of Incoming Calls | N/A | N/A | Std | Std | Std |
| BCOCIU/BCOCIS Analog CO Interface with Caller ID | No | No | No | Yes | Yes |
| BSTCIU 8-port Analog Station Interface with Caller ID | No | No | No | Yes | Yes |
| Caller ID Call History All Calls | N/A | N/A | Opt | Opt | Opt |
| Caller ID Call Waiting with CLASS Information | N/A | N/A | Opt | Opt | Opt |
| Caller ID ISDN Caller ID Names | N/A | N/A | Opt | Opt | Opt |
| Call Forward - System wide | N/A | N/A | Std | Std | Std |
| Call Forward - Station - Any Call | N/A | N/A | Std | Std | Std |
| Call Forward - Station - Line Calls Only | N/A | N/A | Std | Std | Std |
| Cancel Button | N/A | N/A | Std | Std | Std |
| CO Line Separate Incoming & Outgoing Line Groups | N/A | N/A | Std | Std | Std |
| CO Lines Can Be in Multiple Incoming or Outgoing Line Groups | N/A | N/A | Std | Std | Std |
| Conference Split & Join | N/A | N/A | Std | Std | Std |
| Conference Meet-me Scheduling with Web Collaboration | N/A | N/A | N/A | Opt | Opt |
| Conference with Web Collaboration | N/A | N/A | N/A | Opt | Opt |
| CTI CSTA LAN Connection for System OAI | N/A | N/A | Opt | Opt | Opt |
| CTI CSTA Pilot DN | N/A | N/A | Std | Std | Std |
| CTI PC Interface Unit for DKT (BPCI) with Outlook Integration | Opt | Opt | Opt | Opt | Opt |
| Database storage/upgrade SmartMedia flash memory card | N/A | N/A | Std | N/A | N/A |
| Database storage/upgrade Secure Digital flash memory card | N/A | N/A | N/A | Std | Std |
| Emergency Call to Any Destination | N/A | N/A | Std | Std | Std |
| Hot Dialpad | N/A | N/A | Std | Std | Std |
| IP Attendant Console support | N/A | N/A | Opt | Opt | Opt |
| IP Telephone support | N/A | N/A | Opt | Opt | Opt |
| IP Telephones VLAN support | N/A | N/A | N/A | Opt | Opt |
| IP Hot Desking with IP User Mobility | N/A | N/A | N/A | Opt | Opt |
| IP SIP Trunking | N/A | N/A | N/A | Opt | Opt |
| LCD Large Display | N/A | N/A | Std | Std | Std |
| LCD Voice Mail Message Count | N/A | N/A | Std | Std | Std |
| LCD Dial by Name Directory on Small LCD | N/A | N/A | N/A | Std | Std |
| LCD Spanish Prompts | N/A | N/A | Std | Std | Std |
| Lost Call Treatment | N/A | N/A | Std | Std | Std |
| Media Application Server (MAS) compatibility | N/A | N/A | Opt | Opt | Opt |
| - FeatureFlex | N/A | N/A | Opt | Opt | Opt |
| - Video Communication Solution (VCS) | N/A | N/A | Opt | Opt | Opt |
| - Unified Administration CIX/SES (eManager) | N/A | N/A | Opt | Opt | Opt |
| - Personal Administration (My Phone Manager) | N/A | N/A | Opt | Opt | Opt |
| - Call Manager UC application | N/A | N/A | Opt | Opt | Opt |
| Message Waiting Stutter Dial Tone | N/A | N/A | Std | Std | Std |
| Microsoft Exchange 2007 UM Integration | N/A | N/A | N/A | Opt | Opt |
| Networking - Alternate Routing/Hop-off | N/A | N/A | Opt | Opt | Opt |
| Networking - Coordinated Numbering/Dialing Plan | N/A | N/A | Opt | Opt | Opt |
| Networking - Centralized Voice Mail | N/A | N/A | Opt | Opt | Opt |
| Networking - Centralized Attendant | N/A | N/A | Opt | Opt | Opt |
| Networking - IP | N/A | N/A | Opt | Opt | Opt |
| Networking - Centralized Networked SMDR | N/A | N/A | N/A | Opt | Opt |
| Night Service Auto Scheduled Activation | N/A | N/A | Std | Std | Std |
| Paging - Emergency Page | N/A | N/A | Std | Std | Std |
| System Fault Finding & Diagnostics | N/A | N/A | Std | Std | Std |
| - Alarm Indication of System Faults | N/A | N/A | Std | Std | Std |
| - Automatic Fault Recovery | N/A | N/A | Std | Std | Std |
| - Fault Detection and Error Logs | N/A | N/A | Std | Std | Std |
| - Maintenance & Administration via LAN | N/A | N/A | Std | Std | Std |
| - Self Test | N/A | N/A | Std | Std | Std |
| - System Trace | N/A | N/A | Std | Std | Std |
| - SMTP Traps for Alarm Notification | N/A | N/A | N/A | Std | Std |
| - Traffic Measurement and Reporting | N/A | N/A | N/A | Std | Std |
| Through Dialing from Station | N/A | N/A | Std | Std | Std |
| Uniform Call Distribution (UCD) | N/A | N/A | N/A | Std | Std |
| Voice Mail Integration Enhancements | N/A | N/A | Std | Std | Std |
| - Call Record to Voice Mail | N/A | N/A | Opt | Opt | Opt |
| - Transfer Direct to Voice Mailbox | N/A | N/A | Opt | Opt | Opt |
| - Voice Mail LCD Prompt/Soft Key Operation | N/A | N/A | Opt | Opt | Opt |
| - Voice Mail Monitor & Retrieve (FeatureFlex feature) | N/A | N/A | Opt | Opt | Opt |
| - Voice mail conference | N/A | N/A | Opt | Opt | Opt |
| ### Maximum Capacities | DK424 | DK424i | CTX670 | CIX670 | CIX1200 |
| Processor Models | 4 | 4 | 1 | 1 | 1 |
| Trunk/Station Ports | 432 | 432 | 672 | 672 | 1,152 |
| Trunk Lines | 200 | 200 | 264 | 264 | 440 |
| IP Telephones | N/A | N/A | 560 | 560 | 1,000 |
| Digital Telephones | 336 | 336 | 560 | 560 | 952 |
| Analog Telephones | 328 | 328 | 544 | 544 | 944 |
| OPXs | 328 | 328 | 544 | 544 | 792 |
| PC Interface Units | 280 | 280 | 462 | 462 | 792 |
| DSS Consoles 60-button | 8 | 8 | 16 | 16 | 24 |
| Add-On Modules 20-button | 200 | 200 | 560 | 560 | 800 |
| NIC 10base-T Interface Port | N/A | N/A | 1 | 1 | 1 |
| Power Failure Telephones | 328 | 328 | 552 | 552 | 552 |
| CO Line Groups | 16 | 16 | 256 | 256 | 440 |
| CO Line Groups - Incoming | N/A | N/A | 128 | 128 | 220 |
| CO Line Groups - Outgoing | N/A | N/A | 128 | 128 | 220 |
| Call Pickup Groups | 20 | 20 | 32 | 32 | 48 |
| Conference Parties | 4 | 4 | 8 | 8 | 8 |
| Simultaneous Conferences | 14 | 14 | 30 | 30 | 40 |
| Trunk-to-trunk conferences | 20 | 20 | 30 | 30 | 40 |
| Station Speed Dial | 40 | 40 | 100 | 100 | 100 |
| Speed Dial Number Digits | 20 | 20 | 32 | 32 | 32 |
| Speed Dial Number Digits Chained | 37 | 37 | 64 | 64 | 64 |
| Multiple Directory Numbers (PDN) | 336 | 336 | 560 | 560 | 1,000 |
| Phantom Directory Numbers (PhDN) | 336 | 336 | 2,240 | 2,240 | 4,000 |
| DNIS Numbers | 500 | 500 | 2,000 | 2,000 | 3,000 |
| DNIS/DID Network Routing Numbers | 500 | 500 | 1,000 | 1,000 | 1,500 |
| External Page Zones | 8 | 8 | 16 | 16 | 16 |
| Internal Page Groups | 9 | 9 | 16 | 16 | 24 |
| Park Orbit Zones | 20 | 20 | 64 | 64 | 96 |
| Station Hunting Groups | 16 | 16 | 640 | 640 | 1,100 |
| Distributed Hunt Group | 16 | 16 | 640 | 640 | 1,100 |
| Station Ring Tones | 3 | 3 | 4 | 10 | 10 |
| Toll Restriction Levels | 8 | 8 | 16 | 16 | 16 |
| Tenants | 4 | 4 | 8 | 8 | 8 |
| Music-on-Hold Interfaces | 1 | 1 | 15 | 15 | 15 |
| Verified Account Codes | 300 | 300 | 1,000 | 1,000 | 4,000 |